- ipmitool工具常用命令汇总

目录

ipmitool工具常用命令汇总	1

1.工具的安装	3

2.命令的help	3

3.查看管理口的IP地址	10

4.管理底盘chassis配置	11

4.1查看底盘配置汇总	11

4.2管理电源	13

4.3管理时间间隔	14

4.4管理电源恢复策略	15

4.5查看上次重启原因	15

4.6查询上电时间	15

4.7管理启动项	16

4.8设置启动项的参数	16

5.使用delloem进行管理	18

5.1配置lcd	19

5.2查看MAC地址	21

5.3设置lan的模式	22

5.4设置背板led的状态	24

5.5管理电源监控	25

6.查看传感器数据sensor	26

6.1查看当前系统所有的传感器数据	27

6.2设置传感器的告警阈值	27

6.3查看某一传感器的状态信息	28

6.4查看某一个传感器的当前值	30

7.管理传感器仓库sdr	30

7.1查看传感器仓库中的信息列表	33

7.2查看传感器的类型和指定类型的数据	35

7.3查看指定传感器的配置信息	39

7.4查看传感器仓库sdr的配置信息	40

8.查看系统事件sel日志	41

8.1查看sel的状态信息	42

8.2查看sel的中的日志信息	42

# 1.工具的安装

yum install -y OpenIPMI ipmitool

开启服务

/etc/init.d/ipmi status

/etc/init.d/ipmi start

[root@cell83 ~]# ipmitool -V

ipmitool version 1.8.18

# 2.命令的help

明细的命令参考man ipmitool

或者附件ipmitool.help

[root@cell83 ~]# ipmitool -h

ipmitool version 1.8.18

usage: ipmitool [options...] <command>

-h             This help

-V             Show version information

-v             Verbose (can use multiple times)

-c             Display output in comma separated format

-d N           Specify a /dev/ipmiN device to use (default=0)

-I intf        Interface to use

-H hostname    Remote host name for LAN interface

-p port        Remote RMCP port [default=623]

-U username    Remote session username

-f file        Read remote session password from file

-z size        Change Size of Communication Channel (OEM)

-S sdr         Use local file for remote SDR cache

-D tty:b[:s]   Specify the serial device, baud rate to use

and, optionally, specify that interface is the system one

-4             Use only IPv4

-6             Use only IPv6

-a             Prompt for remote password

-Y             Prompt for the Kg key for IPMIv2 authentication

-e char        Set SOL escape character

-C ciphersuite Cipher suite to be used by lanplus interface

-k key         Use Kg key for IPMIv2 authentication

-y hex_key     Use hexadecimal-encoded Kg key for IPMIv2 authentication

-L level       Remote session privilege level [default=ADMINISTRATOR]

Append a '+' to use name/privilege lookup in RAKP1

-A authtype    Force use of auth type NONE, PASSWORD, MD2, MD5 or OEM

-P password    Remote session password

-E             Read password from IPMI_PASSWORD environment variable

-K             Read kgkey from IPMI_KGKEY environment variable

-m address     Set local IPMB address

-b channel     Set destination channel for bridged request

-t address     Bridge request to remote target address

-B channel     Set transit channel for bridged request (dual bridge)

-T address     Set transit address for bridge request (dual bridge)

-l lun         Set destination lun for raw commands

-o oemtype     Setup for OEM (use 'list' to see available OEM types)

-O seloem      Use file for OEM SEL event descriptions

-N seconds     Specify timeout for lan [default=2] / lanplus [default=1] interface

-R retry       Set the number of retries for lan/lanplus interface [default=4]

Interfaces:

open          Linux OpenIPMI Interface [default]

imb           Intel IMB Interface

lan           IPMI v1.5 LAN Interface

lanplus       IPMI v2.0 RMCP+ LAN Interface

serial-terminal  Serial Interface, Terminal Mode

serial-basic  Serial Interface, Basic Mode

usb           IPMI USB Interface(OEM Interface for AMI Devices)

Commands:

raw           Send a RAW IPMI request and print response

i2c           Send an I2C Master Write-Read command and print response

spd           Print SPD info from remote I2C device

lan           Configure LAN Channels

chassis       Get chassis status and set power state

power         Shortcut to chassis power commands

event         Send pre-defined events to MC

mc            Management Controller status and global enables

sdr           Print Sensor Data Repository entries and readings

sensor        Print detailed sensor information

fru           Print built-in FRU and scan SDR for FRU locators

gendev        Read/Write Device associated with Generic Device locators sdr

sel           Print System Event Log (SEL)

pef           Configure Platform Event Filtering (PEF)

sol           Configure and connect IPMIv2.0 Serial-over-LAN

tsol          Configure and connect with Tyan IPMIv1.5 Serial-over-LAN

isol          Configure IPMIv1.5 Serial-over-LAN

user          Configure Management Controller users

channel       Configure Management Controller channels

session       Print session information

dcmi          Data Center Management Interface

nm            Node Manager Interface

sunoem        OEM Commands for Sun servers

kontronoem    OEM Commands for Kontron devices

picmg         Run a PICMG/ATCA extended cmd

fwum          Update IPMC using Kontron OEM Firmware Update Manager

firewall      Configure Firmware Firewall

delloem       OEM Commands for Dell systems

shell         Launch interactive IPMI shell

exec          Run list of commands from file

set           Set runtime variable for shell and exec

hpm           Update HPM components using PICMG HPM.1 file

ekanalyzer    run FRU-Ekeying analyzer using FRU files

ime           Update Intel Manageability Engine Firmware

vita          Run a VITA 46.11 extended cmd

lan6          Configure IPv6 LAN Channels

# 3.查看管理口的IP地址

[root@cell83 ~]# ipmitool -I open lan print

[root@cell83 ~]# ipmitool -I open lan print | grep IP

IP Address Source       : Static Address

IP Address              : 192.168.0.133

IP Header               : TTL=0x40 Flags=0x40 Precedence=0x00 TOS=0x10

Default Gateway IP      : 192.168.0.1

Backup Gateway IP       : 0.0.0.0

# 4.管理底盘chassis配置

[root@cell83 ~]# ipmitool -I open chassis help

Chassis Commands:  status, power, identify, policy, restart_cause, poh, bootdev, bootparam, selftest

## 4.1查看底盘配置汇总

[root@cell83 ~]# ipmitool -I open chassis status

System Power         : on

Power Overload       : false

Power Interlock      : inactive

Main Power Fault     : false

Power Control Fault  : false

Power Restore Policy : previous

Last Power Event     :

Chassis Intrusion    : inactive

Front-Panel Lockout  : inactive

Drive Fault          : false

Cooling/Fan Fault    : false

Sleep Button Disable : not allowed

Diag Button Disable  : allowed

Reset Button Disable : not allowed

Power Button Disable : allowed

Sleep Button Disabled: false

Diag Button Disabled : true

Reset Button Disabled: false

Power Button Disabled: false

## 4.2管理电源

查看当前电源状态

[root@cell83 ~]# ipmitool -I open chassis  power  status

Chassis Power is on

打开电源

ipmitool -I open chassis  power  on

关闭电源

ipmitool -I open chassis  power  off

重启电源

ipmitool -I open chassis  power  cycle

重置电源

ipmitool -I open chassis  power  reset

其他配置

ipmitool -I open chassis  power  diag

ipmitool -I open chassis  power  soft

## 4.3管理时间间隔

查询时间间隔：

[root@cell83 ~]# ipmitool -I open chassis identify

Chassis identify interval: default (15 seconds)

设置时间间隔：

[root@cell83 ~]# ipmitool -I open chassis identify help

chassis identify <interval>

default is 15 seconds

0 to turn off

force to turn on indefinitely

## 4.4管理电源恢复策略

[root@cell83 ~]# ipmitool -I open chassis policy help

chassis policy <state>

list        : return supported policies

always-on   : turn on when power is restored

previous    : return to previous state when power is restored

always-off  : stay off after power is restored

## 4.5查看上次重启原因

[root@cell83 ~]# ipmitool -I open chassis restart_cause

System restart cause: unknown

## 4.6查询上电时间

[root@cell83 ~]# ipmitool -I open chassis poh

Get Chassis Power-On-Hours failed: Invalid command

## 4.7管理启动项

[root@cell83 ~]# ipmitool -I open chassis bootdev help

bootdev <device> [clear-cmos=yes|no]

bootdev <device> [options=help,...]

none  : Do not change boot device order

pxe   : Force PXE boot

disk  : Force boot from default Hard-drive

safe  : Force boot from default Hard-drive, request Safe Mode

diag  : Force boot from Diagnostic Partition

cdrom : Force boot from CD/DVD

bios  : Force boot into BIOS Setup

floppy: Force boot from Floppy/primary removable media

## 4.8设置启动项的参数

[root@cell83 ~]# ipmitool -I open chassis bootparam help

bootparam get <param #>

bootparam set bootflag <device> [options=...]

Legal devices are:

none        : No override

force_pxe   : Force PXE boot

force_disk  : Force boot from default Hard-drive

force_safe  : Force boot from default Hard-drive, request Safe Mode

force_diag  : Force boot from Diagnostic Partition

force_cdrom : Force boot from CD/DVD

force_bios  : Force boot into BIOS Setup

Legal options are:

help    : print this message

PEF     : Clear valid bit on reset/power cycle cause by PEF

timeout : Automatically clear boot flag valid bit on timeout

watchdog: Clear valid bit on reset/power cycle cause by watchdog

reset   : Clear valid bit on push button reset/soft reset

power   : Clear valid bit on power up via power push button or wake event

Any Option may be prepended with no- to invert sense of operation

# 5.使用delloem进行管理

查看帮助

[root@cell83 ~]# ipmitool -I open delloem help

usage: delloem <command> [option...]

commands:

lcd

mac

lan

setled

powermonitor

vFlash

For help on individual commands type:

delloem <command> help

## 5.1配置lcd

[root@cell83 ~]# ipmitool -I open delloem lcd help

Generic DELL HW:

lcd set {none}|{default}|{custom <text>}

Set LCD text displayed during non-fault conditions

iDRAC 11g or iDRAC 12g or  iDRAC 13g :

lcd set {mode}|{lcdqualifier}|{errordisplay}

Allows you to set the LCD mode and user-defined string.

lcd set mode {none}|{modelname}|{ipv4address}|{macaddress}|

{systemname}|{servicetag}|{ipv6address}|{ambienttemp}

{systemwatt }|{assettag}|{userdefined}<text>

Allows you to set the LCD display mode to any of the preceding

parameters

lcd set lcdqualifier {watt}|{btuphr}|{celsius}|{fahrenheit}

Allows you to set the unit for the system ambient temperature mode.

lcd set errordisplay {sel}|{simple}

Allows you to set the error display.

lcd info

Show LCD text that is displayed during non-fault conditions

lcd set vkvm{active}|{inactive}

Set vKVM active and inactive, message will be displayed on lcd

when vKVM is active and vKVM session is in progress

lcd set frontpanelaccess {viewandmodify}|{viewonly}|{disabled}

Set LCD mode to view and modify, view only or disabled

lcd status

Show LCD Status for vKVM display<active|inactive>

and Front Panel access mode {viewandmodify}|{viewonly}|{disabled}

## 5.2查看MAC地址

[root@cell83 ~]# ipmitool -I open delloem mac help

mac list

Lists the MAC address of LOMs

mac get <NIC number>

Shows the MAC address of specified LOM. 0-7 System LOM, 8- DRAC/iDRAC.

[root@cell83 ~]# ipmitool -I open delloem mac list

System LOMs

NIC Number	MAC Address		Status

0		b8:2a:72:db:56:2a	Enabled

1		b8:2a:72:db:56:2b	Enabled

2		b8:2a:72:db:56:28	Enabled

3		b8:2a:72:db:56:29	Enabled

iDRAC7 MAC Address b0:83:fe:c9:6d:31

## 5.3设置lan的模式

[root@cell83 ~]# ipmitool -I open delloem lan help

lan set <Mode>

sets the NIC Selection Mode :

on iDRAC12g OR iDRAC13g  :

dedicated, shared with lom1, shared with lom2,shared with lom3,shared

with lom4,shared with failover lom1,shared with failover lom2,shared

with failover lom3,shared with failover lom4,shared with Failover all

loms, shared with Failover None).

on other systems :

dedicated, shared, shared with failover lom2,

shared with Failover all loms.

lan get

on iDRAC12g or iDRAC13g  :

returns the current NIC Selection Mode (dedicated, shared with lom1, shared

with lom2, shared with lom3, shared with lom4,shared with failover lom1,

shared with failover lom2,shared with failover lom3,shared with failover

lom4,shared with Failover all loms,shared with Failover None).

on other systems :

dedicated, shared, shared with failover,

lom2, shared with Failover all loms.

lan get active

returns the current active NIC (dedicated, LOM1, LOM2, LOM3, LOM4).

## 5.4设置背板led的状态

[root@cell83 ~]# ipmitool -I open delloem setled help

setled <b:d.f> <state..>

Set backplane LED state

b:d.f = PCI Bus:Device.Function of drive (lspci format)

state = present|online|hotspare|identify|rebuilding|

fault|predict|critical|failed

## 5.5管理电源监控

[root@cell83 ~]# ipmitool -I open delloem powermonitor help

powermonitor

Shows power tracking statistics

powermonitor clear cumulativepower

Reset cumulative power reading

powermonitor clear peakpower

Reset peak power reading

powermonitor powerconsumption

Displays power consumption in <watt|btuphr>

powermonitor powerconsumptionhistory <watt|btuphr>

Displays power consumption history

powermonitor getpowerbudget

Displays power cap in <watt|btuphr>

powermonitor setpowerbudget <val><watt|btuphr|percent>

Allows user to set the  power cap in <watt|BTU/hr|percentage>

powermonitor enablepowercap

To enable set power cap

powermonitor disablepowercap

To disable set power cap

# 6.查看传感器数据sensor

[root@cell83 ~]# ipmitool -I open sensor help

Sensor Commands:  list thresh get reading

## 6.1查看当前系统所有的传感器数据

[root@cell83 ~]# ipmitool -I open sensor list

## 6.2设置传感器的告警阈值

[root@cell83 ~]# ipmitool -I open sensor thresh help

sensor thresh <id> <threshold> <setting>

id        : name of the sensor for which threshold is to be set

threshold : which threshold to set

unr = upper non-recoverable

ucr = upper critical

unc = upper non-critical

lnc = lower non-critical

lcr = lower critical

lnr = lower non-recoverable

setting   : the value to set the threshold to

sensor thresh <id> lower <lnr> <lcr> <lnc>

Set all lower thresholds at the same time

sensor thresh <id> upper <unc> <ucr> <unr>

Set all upper thresholds at the same time

## 6.3查看某一传感器的状态信息

[root@cell83 ~]# ipmitool -I open sensor get help

sensor get <id> ... [id]

id        : name of desired sensor

查看温度传感器

[root@cell83 ~]# ipmitool -I open sensor get Temp

Locating sensor record...

Sensor ID              : Temp (0xe)

Entity ID             : 3.1

Sensor Type (Threshold)  : Temperature

Sensor Reading        : 48 (+/- 1) degrees C

Status                : ok

Lower Non-Recoverable : na

Lower Critical        : 3.000

Lower Non-Critical    : 8.000

Upper Non-Critical    : 79.000

Upper Critical        : 84.000

Upper Non-Recoverable : na

Positive Hysteresis   : 1.000

Negative Hysteresis   : 1.000

Assertion Events      :

Assertions Enabled    :

## 6.4查看某一个传感器的当前值

[root@cell83 ~]# ipmitool -I open sensor reading help

sensor reading <id> ... [id]

id        : name of desired sensor

查看温度传感器

[root@cell83 ~]# ipmitool -I open sensor reading Temp

Temp             | 47

# 7.管理传感器仓库sdr

[root@cell83 ~]# ipmitool -I open sdr help

usage: sdr <command> [options]

list | elist [option]

all           All SDR Records

full          Full Sensor Record

compact       Compact Sensor Record

event         Event-Only Sensor Record

mcloc         Management Controller Locator Record

fru           FRU Locator Record

generic       Generic Device Locator Record

type [option]

<Sensor_Type> Retrieve the state of specified sensor.

Sensor_Type can be specified either as

a string or a hex value.

list          Get a list of available sensor types

get <Sensor_ID>

Retrieve state of the first sensor matched by Sensor_ID

info

Display information about the repository itself

entity <Entity_ID>[.<Instance_ID>]

Display all sensors associated with an entity

dump <file>

Dump raw SDR data to a file

fill <option>

sensors       Creates the SDR repository for the current

configuration

nosat         Creates the SDR repository for the current

configuration, without satellite scan

file <file>   Load SDR repository from a file

range <range> Load SDR repository from a provided list

or range. Use ',' for list or '-' for

range, eg. 0x28,0x32,0x40-0x44

## 7.1查看传感器仓库中的信息列表

[root@cell83 ~]# ipmitool -I open sdr elist help

usage: sdr elist [all|full|compact|event|mcloc|fru|generic]

=>查看仓库中所有的信息：elist和list的格式内容上略有不同

[root@cell83 ~]# ipmitool -I open sdr elist all

[root@cell83 ~]# ipmitool -I open sdr list all

=>查看传感器的数据

[root@cell83 ~]# ipmitool -I open sdr elist full

Fan1 RPM         | 30h | ok  |  7.1 | 6240 RPM

Fan2 RPM         | 31h | ok  |  7.1 | 6240 RPM

Fan3 RPM         | 32h | ok  |  7.1 | 6240 RPM

Fan4 RPM         | 33h | ok  |  7.1 | 6120 RPM

Fan5 RPM         | 34h | ok  |  7.1 | 6240 RPM

Fan6 RPM         | 35h | ok  |  7.1 | 6240 RPM

Inlet Temp       | 04h | ok  |  7.1 | 35 degrees C

Exhaust Temp     | 01h | ok  |  7.1 | 40 degrees C

Temp             | 0Eh | ok  |  3.1 | 47 degrees C

Temp             | 0Fh | ok  |  3.2 | 46 degrees C

Current 1        | 6Ah | ok  | 10.1 | 0.80 Amps

Current 2        | 6Bh | ns  | 10.2 | No Reading

Voltage 1        | 6Ch | ok  | 10.1 | 236 Volts

Voltage 2        | 6Dh | ns  | 10.2 | No Reading

Pwr Consumption  | 77h | ok  |  7.1 | 168 Watts

说明：

Fan   ----表示风扇

Inlet Temp  ----进口温度

Exhaust Temp  ----出口温度

Temp  ----CPU的温度

Current  --电流

Voltage  ---电压

Pwr Consumption	 ----功率

## 7.2查看传感器的类型和指定类型的数据

查看传感器有哪些类型

[root@cell83 ~]# ipmitool -I open sdr type list

Sensor Types:

Temperature               (0x01)   Voltage                   (0x02)

Current                   (0x03)   Fan                       (0x04)

Physical Security         (0x05)   Platform Security         (0x06)

Processor                 (0x07)   Power Supply              (0x08)

Power Unit                (0x09)   Cooling Device            (0x0a)

Other                     (0x0b)   Memory                    (0x0c)

Drive Slot / Bay          (0x0d)   POST Memory Resize        (0x0e)

System Firmwares          (0x0f)   Event Logging Disabled    (0x10)

Watchdog1                 (0x11)   System Event              (0x12)

Critical Interrupt        (0x13)   Button                    (0x14)

Module / Board            (0x15)   Microcontroller           (0x16)

Add-in Card               (0x17)   Chassis                   (0x18)

Chip Set                  (0x19)   Other FRU                 (0x1a)

Cable / Interconnect      (0x1b)   Terminator                (0x1c)

System Boot Initiated     (0x1d)   Boot Error                (0x1e)

OS Boot                   (0x1f)   OS Critical Stop          (0x20)

Slot / Connector          (0x21)   System ACPI Power State   (0x22)

Watchdog2                 (0x23)   Platform Alert            (0x24)

Entity Presence           (0x25)   Monitor ASIC              (0x26)

LAN                       (0x27)   Management Subsys Health  (0x28)

Battery                   (0x29)   Session Audit             (0x2a)

Version Change            (0x2b)   FRU State                 (0x2c)

查看指定传感器的数据

如查看温度：

[root@cell83 ~]# ipmitool -I open sdr type Temperature

Inlet Temp       | 04h | ok  |  7.1 | 35 degrees C

Exhaust Temp     | 01h | ok  |  7.1 | 40 degrees C

Temp             | 0Eh | ok  |  3.1 | 46 degrees C

Temp             | 0Fh | ok  |  3.2 | 47 degrees C

如查看电源的个数1：有一个电源

[root@cell83 dvd]# ipmitool -I open sdr type "Power Supply"

PS Redundancy    | 74h | ns  |  7.1 | No Reading

Status           | 62h | ok  | 10.1 | Presence detected

Status           | 63h | ns  | 10.2 | Disabled

如查看电源的个数2：有四个电源

[root@db04 ~]# ipmitool -I open sdr type "Power Supply"

PS Redundancy    | 74h | ok  |  7.1 | Redundancy Lost

Status           | 62h | ok  | 10.1 | Presence detected

Status           | 63h | ok  | 10.2 | Presence detected, Power Supply AC lost

Status           | 52h | ok  | 10.3 | Presence detected, Config Error, Power Supply Inactive

Status           | 53h | ok  | 10.4 | Presence detected, Config Error, Power Supply Inactive

## 7.3查看指定传感器的配置信息

[root@cell83 ~]# ipmitool -I open sdr get Temp

Sensor ID              : Temp (0xe)

Entity ID             : 3.1 (Processor)

Sensor Type (Threshold)  : Temperature (0x01)

Sensor Reading        : 47 (+/- 1) degrees C

Status                : ok

Nominal Reading       : 23.000

Normal Minimum        : 11.000

Normal Maximum        : 69.000

Upper critical        : 83.000

Upper non-critical    : 77.000

Lower critical        : 3.000

Lower non-critical    : 8.000

Positive Hysteresis   : 1.000

Negative Hysteresis   : 1.000

Minimum sensor range  : Unspecified

Maximum sensor range  : Unspecified

Event Message Control : Per-threshold

Readable Thresholds   : No Thresholds

Settable Thresholds   : No Thresholds

Assertion Events      :

Assertions Enabled    :

## 7.4查看传感器仓库sdr的配置信息

[root@cell83 ~]# ipmitool -I open sdr info

SDR Version                         : 0x51

Record Count                        : 170

Free Space                          : 1836 bytes

Most recent Addition                : 02/07/2106 06:28:15

Most recent Erase                   : 02/07/2106 06:28:15

SDR overflow                        : no

SDR Repository Update Support       : modal

Delete SDR supported                : no

Partial Add SDR supported           : no

Reserve SDR repository supported    : yes

SDR Repository Alloc info supported : no

# 8.查看系统事件sel日志

[root@cell83 ~]# ipmitool -I open sel help

SEL Commands:  info clear delete list elist get add time save readraw writeraw interpret

## 8.1查看sel的状态信息

[root@cell83 ~]# ipmitool -I open sel info help

SEL Information

Version          : 1.5 (v1.5, v2 compliant)

Entries          : 740

Free Space       : 4544 bytes

Percent Used     : 72%

Last Add Time    : 08/27/2018 08:48:29

Last Del Time    : 09/13/2014 11:16:12

Overflow         : false

Supported Cmds   : 'Reserve'

## 8.2查看sel的中的日志信息

ipmitool -I open sel  list | elist

When  this  command  is invoked without arguments, the entire contents of the System Event Log are displayed.  If invoked as elist (extended list) it will also use the Sensor Data Record entries to display the sensor ID for the sensor that caused each event.  Note this can take a long time over the system interface.

<count> | first <count>

Displays the first count (least-recent) entries in the SEL.  If count is zero, all entries are displayed.

last <count>

Displays the last count (most-recent) entries in the SEL.  If count is zero, all entries are displayed.

查看前5条

[root@cell83 ~]# ipmitool -I open sel elist first 5

1 | 09/13/2014 | 11:16:12 | Event Logging Disabled SEL | Log area reset/cleared | Asserted

2 | 10/15/2014 | 13:42:04 | Physical Security Intrusion | General Chassis intrusion () | Asserted

3 | 10/15/2014 | 13:42:09 | Physical Security Intrusion | General Chassis intrusion () | Deasserted

4 | 10/16/2014 | 12:08:20 | Physical Security Intrusion | General Chassis intrusion () | Asserted

5 | 10/16/2014 | 12:08:49 | Physical Security Intrusion | General Chassis intrusion () | Deasserted

查看后5条

[root@cell83 ~]# ipmitool -I open sel elist last 5

2e0 | 08/02/2018 | 01:45:10 | Physical Security Intrusion | General Chassis intrusion () | Deasserted

2e1 | 08/04/2018 | 04:11:34 | OS Boot | Installation started | Asserted

2e2 | 08/04/2018 | 04:21:03 | OS Boot | Installation completed | Asserted

2e3 | 08/27/2018 | 08:48:24 | Physical Security Intrusion | General Chassis intrusion () | Asserted

2e4 | 08/27/2018 | 08:48:29 | Physical Security Intrusion | General Chassis intrusion () | Deasserted单纯的课本内容，并不能满足学生的需要，通过补充，达到内容的完善

教育之通病是教用脑的人不用手，不教用手的人用脑，所以一无所能。教育革命的对策是手脑联盟，结果是手与脑的力量都可以大到不可思议。